Windows CE 5.0 MCS7830 Diagnostic Utility User Manual

	Diagnostic Utility is a Command Based Tool which runs some diagnostic Tests on the MCS7830.

	See the Usage Options and the Sample Output below.

Usage

Diagnosticutility r - It writes and reads back the data to the HIF & PHY registers.

Diagnosticutility i pktsize Iterations - It Does the Internal Loopback Test. pktsize should be less than 1515. pktsize and Iterations are integer decimal values.

Diagnosticutility e pktsize Iterations - It Does the External Loopback Test. pktsize should be less than 1515. pktsize and Iterations are integer decimal values. For this test, the External LoopBack Cable should be plugged in. The test will not proceed when the cable is not present.

Diagnosticutility a pktsize Iterations - It Does the Internal Loopback Test & External LoopBack Test. pktsize should be less than 1515. pktsize and Iterations are integer decimal values.

Sample Input/Output

>diagnosticutility.exe r

Start Eprom Programming...

Start Register Test...

Done..

Stop Eprom Programming

>diagnosticutility.exe i 1514 1000

Start Eprom Programming...

Start Internal LoopBack Test...

Done..

Stop Eprom Programming

>diagnosticutility.exe e 1514 1000

Start Eprom Programming...

Start External LoopBack Test...

Done..

Stop Eprom Programming

>diagnosticutility.exe a 1514 1000

Start Eprom Programming...

Start Register Test...

Done..

Start Internal LoopBack Test...

Done..

Start External LoopBack Test...

Done..

Stop Eprom Programming

>Diagnosticutility

USAGE::

1.Register Test	- 'DiagnosticUtility r'

2.Internal LoopBack Test  - 'DiagnosticUtility i pktsize Iterations'

3.External LoopBack Test - 'DiagnosticUtility e pktsize Iterations'

4.All Tests	- 'DiagnosticUtility a pktsize Iterations'